The loudspeaker shall be of the two-way passive type. It shall consist of two reflex-loaded 4” (102mm) neodymium LF/MF drive units and a 1” (25mm) exit, 1.5” (38mm) neodymium ring radiator compression HF drive unit coupled to a rotatable constant-coverage waveguide. The drive units shall be linked via an internal asymmetric passive crossover network providing optimal frequency and phase responses.

The enclosure shall be constructed of premium multi-laminate Birch plywood and coated with black or white impact and weather-resistant polyurethane as standard. RAL colour matching shall be optionally available. The front of the loudspeaker shall be protected by a powder coated hex punched stainless steel grille backed with acoustically transparent fabric. An outdoor (OD) version shall be optionally available with weather-resistant connectors.

The enclosure shall be trapezoidal in shape. The top & bottom shall incorporate a single M8 threaded point each, and the rear of the enclosure shall incorporate a pair of M6 threaded points for attachment of a variety of mounting and flying options.

Two Neutrik® speakON™ NL4MP connectors shall be provided on a rear connector plate for input and enclosure link.

The loudspeaker shall be provided with two interchangeable waveguides, having nominal dispersion patterns of 120° x 70° (wide) and 80° x 70° (narrow), and the HF waveguides can be rotated as required. The frequency response (±3dB) shall be 90Hz – 20kHz when measured on-axis. The maximum continuous SPL shall be 115dB, and the maximum peak SPL (measured with pink noise with crest factor of 6) shall be 121dB. The loudspeaker shall have a power handling of 250W RMS, 500W program. The loudspeaker shall have a nominal impedance of 8 ohms.

The loudspeaker shall have maximum dimensions (HxWxD) of 420 x 114 x 150mm (16.5 x 4.5 x 5.9 inches) and a net weight of 4.7kg (10.3lbs).

The loudspeaker shall be the EM Acoustics R4.